AntiPID

#include "dlab_def.h"

#include <pthread.h>

#include <semaphore.h>

#include <stdio.h>

#include <stdlib.h>

#define MAXS 10000 // Maximum number of samples

pthread_t ControlID;

sem_t data_avail; // Semaphore for synchronization

float theta[MAXS]; // Motor position array

float ref[MAXS];   // Reference input array

float Kp = 300;

float Ti = 0.025;

float Td = 0.00625;

float Tt = 0.01;

float N = 20;

float Fs = 200.0;

float run_time = 10.0;

int motor_number = 2;

float satblk(float v) {

if(v>3.0) {

v=3.0;

}

if(v<-3.0) {

v=-3.0;

}

return v;

}

void *Control(void *arg) {

int k=2;

int no_of_samples = (int)(run_time * Fs);

float motor_position, e[MAXS], u[MAXS], v[MAXS];

float Ts=1/Fs;

float beta=(Td*N)/(Ts*N+Td);

e[0]=ref[0];

e[1]=ref[1];

u[1]=0;

v[1]=0;

while(k < no_of_samples) {

sem_wait(&data_avail);

motor_position = EtoR(ReadEncoder());

e[k] = ref[k] - motor_position;

v[k]=v[k-1]+Kp*((1+beta)*e[k]+(-1+Ts/Ti-2*beta)*e[k-1]+beta*e[k-2]);

v[k]=v[k]+(Ts/Tt)*(u[k-1]-v[k-1]);

u[k]=satblk(v[k]);

DtoA(VtoD(u[k]));

theta[k] = motor_position;

k++;

}

pthread_exit(NULL);

}

int main() {

char selection;

Square(ref, MAXS, Fs, (50 * (3.14159265359 / 180.0)), 0.5, 50);

while(1) {

printf("\nMenu:\n");

printf("r: Run the control algorithm\n");

printf("p: Change value of Kp\n");

printf("i: Change values of Ti\n");

printf("d: Change value of Td\n");

printf("n: Change the value of N\n");

printf("f: Change sample frequency Fs\n");

printf("t: Change total run time Tf\n");

printf("u: Change Change input type (Step/Square)\n");

printf("g: Plot motor Position\n");

printf("h: Save Plot as Postscripts\n");

printf("q: Exit\n");

printf("Enter your selection: ");

scanf(" %c", &selection);

switch(selection) {

case 'r':

sem_init(&data_avail, 0, 0);

Initialize(Fs, motor_number);

pthread_create(&ControlID, NULL, Control, NULL);

pthread_join(ControlID, NULL);

Terminate();

sem_destroy(&data_avail);

break;

case 'p':

printf("\nEnter new proportional gain Kp: ");

scanf("%f", &Kp);

break;

case 'i':

printf("\nEnter new integral time constant Ti: ");

scanf("%f", &Ti);

break;

case 'd':

printf("\nEnter new derivative time constant Td: ");

scanf("%f", &Td);

break;

case 'n':

printf("\nEnter new N: ");

scanf("%f", &N);

break;

case 'f':

printf("\nEnter new sampling frequency Fs: ");

scanf("%f", &Fs);

break;

case 't':

printf("\nEnter new total run time Tf: ");

scanf("%f", &run_time);

break;

case 'u': {

printf("\nSelect reference type (s for Step, q for Square): ");

scanf(" %c", &selection);

if(selection=='s') {

float magnitude;

printf("\nEnter step magnitude (degrees): ");

scanf("%f", &magnitude);

magnitude = magnitude * (3.14159265359 / 180.0);

for (int i = 0; i < MAXS; i++)

ref[i] = magnitude;

}

if(selection=='q') {

float magnitude, frequency, duty_cycle;

printf("\nEnter square wave magnitude (degrees): ");

scanf("%f", &magnitude);

magnitude = magnitude * (3.14159265359 / 180.0);

printf("\nEnter frequency (Hz): ");

scanf("%f", &frequency);

printf("\nEnter duty cycle (%%): ");

scanf("%f", &duty_cycle);

Square(ref, MAXS, Fs, magnitude, frequency, duty_cycle);

}

if(selection!='s'&&selection!='q') {

printf("Invalid reference type\n");

}

break;

}

case 'g':

plot(ref, theta, Fs, ((int)(run_time*Fs)), SCREEN, "Step Response", "Time (s)", "Position (rad)");

break;

case 'h':

plot(ref, theta, Fs, ((int)(run_time*Fs)), PS, "Step Response", "Time (s)", "Position (rad)");

break;

case 'q':

printf("\nAll done");

exit(0);

default:

printf("Invalid selection\n");

}

}

}